WHITMIRE RESEARCH LABORATORIES, INC -- WHITMIRE PT 240 PERMA-DUST -- 6840-00F008594

=====================  Product Identification  =====================

Product ID:WHITMIRE PT 240 PERMA-DUST

MSDS Date:07/01/1987

FSC:6840

NIIN:00F008594

MSDS Number: BGYHS

=== Responsible Party ===

Company Name:WHITMIRE RESEARCH LABORATORIES, INC

Address:3568 TREE COURT IND. BLVD.

City:ST. LOUIS

State:MO

ZIP:63122

Info Phone Num:(314) 225-5371

Emergency Phone Num:(314) 225-5371

Preparer's Name:MICHAEL G. SARLI

CAGE:67184

=== Contractor Identification ===

Company Name:WHITMIRE MICRO GEN RESEARCH LABORATORIES, INC.

Address:3568 TREE COURT IND. BLVD.

Box:City:ST LOUIS

State:MO

ZIP:63122-6620

Country:US

Phone:636-225-5371

CAGE:67184

=============  Composition/Information on Ingredients  =============

Ingred Name:METHYL CHLOROFORM (1,1,1-TRICHLOROEHANE)  (SARA III)

CAS:71-55-6

RTECS #:KJ2975000

Fraction by Wt: <79.5%

Other REC Limits:350 PPM

OSHA PEL:350 PPM/450 STEL

ACGIH TLV:350 PPM/450STEL;9192

EPA Rpt Qty:1000 LBS

DOT Rpt Qty:1000 LBS

Ozone Depleting Chemical:1

Ingred Name:CHLORODIFLUOROMETHANE

CAS:75-45-6

RTECS #:PA6390000

Fraction by Wt: <79.5%

Other REC Limits:1000 PPM

OSHA PEL:1000 PPM

ACGIH TLV:1000PPM; 9192

Ozone Depleting Chemical:2

Ingred Name:BORIC ACID

CAS:10043-35-3

RTECS #:ED4550000

Fraction by Wt: 20.0%

=====================  Hazards Identification  =====================

Routes of Entry: Inhalation:YES  Skin:YES  Ingestion:YES

Reports of Carcinogenicity:NTP:NO    IARC:NO	OSHA:NO

Health Hazards Acute and Chronic:EYES/SKIN: MAY CAUSE IRRITATION.

INHALATION/INGESTION: NAUSEA, VOMITING & DIARRHEA. MINIMAL

ANESTHETIC OR NARCOTIC EFFECTS MAY BE SEEN IN THE RANGE OF 500 TO

1000 PPM 1,1,1-TRICHLOROETHANE.

Explanation of Carcinogenicity:NONE

Effects of Overexposure:EYES/SKIN: MAY CAUSE IRRITATION.

INHALATION/INGESTION: NAUSEA, VOMITING & DIARRHEA. MINIMAL

ANESTHETIC OR NARCOTIC EFFECTS MAY BE SEEN IN THE RANGE OF 5OO TO

1000 PPM 1,1,1-TRICHLORETHANE.

=======================  First Aid Measures  =======================

First Aid:INHALATION: REMOVE PATIENT TO FRESH AIR. APPLY ARTIFICIAL

RESPIRATION IF INDICATED. SKIN: REMOVE CONTAMINATED CLOTHING & WASH

AFFECTED AREAS WITH SOAP & WATER. EYES: FLUSH WITH PLENTY OF WATER.

GET ME DICAL ATTENTION. INGESTION: CALL PHYSICIAN IMMEDIATELY.

GASTRIC LAVAGE INDICATED UNLESS SEMI-CONSCIOUS, UNCONSCIOUS OR

CONVULSIONS. I.V.'S OF ISOTONIC SALINE AND PLASMA ACT AS ANTIDOTE.

=====================  Fire Fighting Measures  =====================

Flash Point:NONE

Extinguishing Media:CO2, DRY CHEMICAL, FOAM

Unusual Fire/Explosion Hazard:CONTENTS UNDER PRESSURE. EXPOSURE TO

TEMPERATURES >130F MAY CAUSE BURSTING.

==================  Accidental Release Measures  ==================

Spill Release Procedures:IF CONTAINER BEGINS TO LEAK (THROUGH PUNCTURE,

ETC.) ALLOW IT TO DISCHARGE COMPLETELY IN A WELL VENTILATED AREA,

THEN DISPOSE OF IN A SAFE PLACE. IN CASE OF SPILLAGE ON SKIN, WASH

THOROUGHLY WITH SOAP  & WATER.

======================	Handling and Storage  ======================

Handling and Storage Precautions:KEEP OUT OF REACH OF CHILDREN. DON'T

STORE NEAR HEAT OR OPEN FLAME. EXPOSURE TO TEMPS >130F MAY CAUSE

BURSTING. CONTENTS UNDER PRESSURE.

Other Precautions:DON'T PUNCTURE. WASH THOROUGHLY AFTER USING. AVOID

CONTAMINATION OF FOOD, UTENSILS & FOOD PREPARATION AREAS.

=============  Exposure Controls/Personal Protection  =============

Respiratory Protection:USE AN APPROVED PESTICIDE DUST RESPIRATOR, WHEN

>TLV EXCEEDED.

Ventilation:PROVIDE ADEQUATE MECHANICAL VENTILATION OF TREATMENT AREA.

Protective Gloves:AVOID SKIN CONTACT

Eye Protection:GOGGLES, FACESHEILD

Other Protective Equipment:NONE REQUIRED

Work Hygienic Practices:WASH THOROUGHLY AFTER USING.

Supplemental Safety and Health

==================  Physical/Chemical Properties  ==================

Vapor Pres:3620

Spec Gravity:0.983

Solubility in Water:4.72%

Appearance and Odor:SPRAY AS A DRY POWDER WITH A SLIGHT SOLVENT ODOR.

Percent Volatiles by Volume:79.0%

=================  Stability and Reactivity Data  =================

Stability Indicator/Materials to Avoid:YES

NONE

Stability Condition to Avoid:DON'T SPRAY INTO OPEN FLAME OR ONTO VERY

HOT SURFACES.

Hazardous Decomposition Products:OPEN FLAME WILL RESULT IN HALOGEN

ACIDS.

====================  Disposal Considerations  ====================

Waste Disposal Methods:AEROSOL CONTAINER ISN'T REFILLABLE. DON'T

ATTEMPT TO RECHARGE. WHEN CONTAINER IS EMPTY, WRAP IN NEWSPAPER &

PLACE IN TRASH COLLECTION. DON'T THROW IN FIRE.

Disclaimer (provided with this information by the compiling agencies):

This information is formulated for use by elements of the Department

of Defense.  The United States of America in no manner whatsoever,

expressly or implied, warrants this information to be accurate and

disclaims all liability for its use.  Any person utilizing this

document should seek competent professional advice to verify and

assume responsibility for the suitability of this information to their

particular situation.